RSPDTS20-520M is a single pull double throw switch, operating over 20MHz to 520MHz and capable of handling 50W. The power handling and bandwidth make it suitable for use in a variety of ECM applications.

| PARAMETER | MINIMUM | MAXIMUM | TYPICAL | COMMENTS |
| --- | --- | --- | --- | --- |
| Frequency | 20MHz | 520MHz |  |  |
| Power Handling |  | 50W |  | 50 ohm impedance |
| Isolation |  |  | 50dB |  |
| Insertion Loss |  | 1dB | 0.5dB |  |
| Switching Speed |  | 3us |  |  |
| Control logic | 0V | 3.3V |  |  |
| Supply Voltage |  |  | 28V |  |
| Dimensions LxWxH |  |  |  | 55mm x 70mm x 15mm |
| Weight |  |  | 400g |  |
| Connectors |  |  |  | SMA Female |
| Operating Temperature | -20°C | +80°C |  | Case Temperature |
| Storage Temperature | -40°C | +85°C |  |  |
| Open/Short Survivability |  | 25W |  |  |

Outline Drawing: Dimensions in inches [mm]

- Truth Table

| Control Input | State |
| --- | --- |
| 1 | J0 – J1. |
| 0 | J0 – J2 |

Note: With control inputs floating default switch state is J0 – J1